Design and Implementation of a Campus/University System Network Design

2025

SUBASH SUBEDI

Table of Contents

1.	Creating a network topology using Cisco Packet Tracer.	3

2.	Hierarchical Network Design.	3

3.	Connecting Networking devices with Correct cabling.	3

4.	Creating VLANs and assigning ports VLAN numbers.	4

5.	Configure Link Aggregation Control Protocol EtherChannel {LACP (802.3ad)}.	9

6.	Subnetting and IP Addressing.	13

7.	Configuring Inter-VLAN Routing (Router on a stick).	14

8.	Configuring DHCP Server (Router as the DHCP Server).	17

9.	Configuring RIPv2 as the routing protocol.	19

10.	Configuring SSH for secure Remote access.	20

11.	Configuring switchport security or Port-Security on the switches.	23

# Design and Implementation of a Campus/University System Network Design (Project #4)

Project #4 Case Study and Requirements

Albion University is a large university which has two campuses situated 20 miles apart. The university’s students and staff are distributed in 4 faculties; these include the faculties of Health and Sciences; Business; Engineering/Computing and Art/Design. Each member of staff has a PC and students have access to PCs in the labs. Create a network topology with the main components to support the following:

- University location.

Main Campus

  - Building A: Administrative staff in the departments of management, HR and finance. The admin staff PCs are distributed in the building offices and it is expected that they will share some networking equipment (Hint: use of VLANs is expected here). The Faculty of Business is also situated in this building

  - Building B: Faculty of Engineering and Computing and Faculty of Art and Design.

  - Building C: Students’ labs and IT department. The IT department hosts the University Web server and other servers - There is also an email server hosted externally on the cloud.

Smaller campus:

  - Faculty of Health and Sciences (staff and students’ labs are situated on separate floors)

- Each department/faculty is expected to be on its own separate IP network.

- The switches should be configured with appropriate VLANs and security settings.

- RIPv2 will be used to provide routing for the routers in the internal network and static routing for the external server.

- The devices in building A will be expected to acquire dynamic IP addresses from a router-based DHCP server.

Configure in Packet Tracer the network with appropriate settings to achieve the connectivity and functionalities specified in the requirements.

Technologies Implemented

# Creating a network topology using Cisco Packet Tracer.

Currently, I am using a combination of Bus Topology and Star Topology.

# Hierarchical Network Design.

# Connecting Networking devices with Correct cabling.

Copper straight cable

Copper Cross Over

Serial cable

# Creating VLANs and assigning ports VLAN numbers.

| ADMIN DEPARTMENT enable configure terminal hostname ADMIN_SWITCH vlan 10 name ADMIN_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 10 ip address 192.168.1.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.1.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 10 no shutdown exit do wr | HR DEPARTMENT enable configure terminal hostname HR_SWITCH vlan 20 name HR_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 20 ip address 192.168.2.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.2.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 20 no shutdown exit do wr |
| --- | --- |
| FINANCE DEPARTMENT enable configure terminal hostname FINANCE_SWITCH vlan 30 name FINANCE_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 30 ip address 192.168.3.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.3.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 30 no shutdown exit do wr | BUSINESS DEPATMENT enable configure terminal hostname BUSINESS_SWITCH vlan 40 name BUSINESS_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 40 ip address 192.168.4.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.4.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 40 no shutdown exit do wr |
| E&C DEPARTMENT enable configure terminal hostname E&C_SWITCH vlan 50 name E&C_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 50 ip address 192.168.5.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.5.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 50 no shutdown exit do wr | E & D DEPARTMENT enable configure terminal hostname A&D_SWITCH vlan 60 name A&D_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 60 ip address 192.168.6.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.6.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 60 no shutdown exit do wr |
| STD-LAP DEPARTMENT enable configure terminal hostname STD-LAP_SWITCH vlan 70 name STD-LAP_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 70 ip address 192.168.7.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.7.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 70 no shutdown exit do wr | SERVER DEPARTMENT enable configure terminal hostname SERVER_SWITCH vlan 80 name SERVER_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 80 ip address 192.168.8.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.8.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 80 no shutdown exit do wr |
| STAFF DEPARTMENT enable configure terminal hostname STAFF_SWITCH vlan 90 name STAFF_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 90 ip address 192.168.9.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.9.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 90 no shutdown exit do wr | STD DEPARTMENT enable configure terminal hostname STD_SWITCH vlan 100 name STD_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit interface vlan 100 ip address 192.168.10.254 255.255.255.0 no shutdown exit ip default-gateway 192.168.10.1 interface range fastEthernet 0/1-24 switchport mode access switchport access vlan 100 no shutdown exit do wr |

| MAIN-MULTILAYER-SWITCH enable configure terminal hostname MAIN_CAMPUS_MULTILAYER_SWITCH vlan 10 name ADMIN_DEPARTMENT exit vlan 20 name HR_DEPARTMENT exit vlan 30 name FINANCE_DEPARTMENT exit vlan 40 name BUSINESS_DEPARTMENT exit vlan 50 name E&C_DEPARTMENT exit vlan 60 name A&D_DEPARTMENT exit vlan 70 name STD_LAP_DEPARTMENT exit vlan 80 name SERVER_DEPARTMENT exit vlan 90 name STAFF_DEPARTMENT exit vlan 100 name STD-PC_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit | BRANCH - CAMPUS enable configure terminal hostname BRANCH_CAMPUS_MULTILAYER_SWITCH vlan 10 name ADMIN_DEPARTMENT exit vlan 20 name HR_DEPARTMENT exit vlan 30 name FINANCE_DEPARTMENT exit vlan 40 name BUSINESS_DEPARTMENT exit vlan 50 name E&C_DEPARTMENT exit vlan 60 name A&D_DEPARTMENT exit vlan 70 name STD_LAP_DEPARTMENT exit vlan 80 name SERVER_DEPARTMENT exit vlan 90 name STAFF_DEPARTMENT exit vlan 100 name STD-PC_DEPARTMENT exit vlan 999 name NATIVE_VLAN exit |
| --- | --- |

# Configure Link Aggregation Control Protocol EtherChannel {LACP (802.3ad)}.

REMINDER
PASSIVE + PASSIVE = NO ETHERCHANNEL

ACTIVE + PASSIVE = ETHERCHANNEL

ACTIVE + ACTIVE = ETHERCHANNEL

WHILE CONFIGURATION LACP FIRST CONIFGRATION L2 SWITCH THEN CONFIGURE ON L3 SWITCH

show etherchannel load-balance

show etherchannel summary

show interfaces status

show etherchannel port-channel

show interfaces GigabitEthernet1/0/2-3 counters

| ADMIN DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 10 switchport nonegotiate exit do wr | HR DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 20 switchport nonegotiate exit do wr |
| --- | --- |
| FINANCE DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 30 switchport nonegotiate exit do wr | BUSINESS DEPATMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 40 switchport nonegotiate exit do wr |
| E&C DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 50 switchport nonegotiate exit do wr | E & D DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 60 switchport nonegotiate exit do wr |
| STD-LAP DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 70 switchport nonegotiate exit do wr | SERVER DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 80 switchport nonegotiate exit do wr |
| STAFF DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 90 switchport nonegotiate exit do wr | STD DEPARTMENT enable configure terminal interface range GigabitEthernet 0/1-2 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 1 mode active no shutdown exit interface port-channel 1 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 10 switchport nonegotiate exit do wr |

| MAIN-MULTILAYER-SWITCH enable configure terminal interface range GigabitEthernet 1/0/3-18 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 2 mode active no shutdown exit interface port-channel 2 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90,100 switchport nonegotiate no shutdown exit do wr | BRANCH CAMPUS enable configure terminal interface range GigabitEthernet 1/0/3-18 description ** THIS TRUNK INTERFACE IS A LACP ETHERCHANNEL ** shutdown channel-protocol lacp channel-group 2 mode active no shutdown exit interface port-channel 2 switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90,100 switchport nonegotiate no shutdown exit do wr |
| --- | --- |

# Subnetting and IP Addressing.

| DEPARTMENT | NETWORK ID | GATEWAYS / STARTING IP | LAST IP / VLAN IP | BROADCAST ID | SUBNET MASK |
| --- | --- | --- | --- | --- | --- |
| ADMIN DEPARTMENT | 192.168.1.0 /24 | 192.168.1.1 | 192.168.1.254 | 192.168.1.255 | 255.255.255.0 |
| HR DEPARTMENT | 192.168.2.0 /24 | 192.168.2.1 | 192.168.2.254 | 192.168.2.255 | 255.255.255.0 |
| FINANCE DEPARTMENT | 192.168.3.0 /24 | 192.168.3.1 | 192.168.3.254 | 192.168.3.255 | 255.255.255.0 |
| BUSINESS DEPARTMENT | 192.168.4.0 /24 | 192.168.4.1 | 192.168.4.254 | 192.168.4.255 | 255.255.255.0 |
| E & C DEPARTMENT | 192.168.5.0 /24 | 192.168.5.1 | 192.168.5.254 | 192.168.5.255 | 255.255.255.0 |
| A & DEPARTMENT | 192.168.6.0 /24 | 192.168.6.1 | 192.168.6.254 | 192.168.6.255 | 255.255.255.0 |
| STD-LAB DEPARTMENT | 192.168.7.0 /24 | 192.168.7.1 | 192.168.7.254 | 192.168.7.255 | 255.255.255.0 |
| SERVER DEPARTMENT | 192.168.8.0 /24 | 192.168.8.1 | 192.168.8.254 | 192.168.8.255 | 255.255.255.0 |
| STAFF DEPARTMENT | 192.168.9.0 /24 | 192.168.9.1 | 192.168.9.254 | 192.168.9.255 | 255.255.255.0 |
| STD DEPARTMENT | 192.168.10.0 /24 | 192.168.10.1 | 192.168.10.254 | 192.168.10.255 | 255.255.255.0 |

| EMAIL SERVER | 20.0.0.0/30 | 20.0.0.2 | 20.0.0.1 {GATEWAYS} |  | 255.255.255.252 |
| --- | --- | --- | --- | --- | --- |
| CLOUD ROUTER | 10.10.10.8/30 | 10.10.10.9 | 10.10.10.10 | 10.10.10.11 | 255.255.255.252 |
| MAIN CAMPUS ROUTER | 10.10.10.0/30 | 10.10.10.1 | 10.10.10.2 | 10.10.10.3 | 255.255.255.252 |
| BRANCH CAMPUS ROUTER | 10.10.10.4/30 | 10.10.10.5 | 10.10.10.6 | 10.10.10.7 | 255.255.255.252 |

# Configuring Inter-VLAN Routing (Router on a stick).

| MAIN-MULTILAYER-SWITCH interface gigabitEthernet 0/0/0.10 encapsulation dot1Q 10 ip address 192.168.1.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.20 encapsulation dot1Q 20 ip address 192.168.2.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.30 encapsulation dot1Q 30 ip address 192.168.3.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.40 encapsulation dot1Q 40 ip address 192.168.4.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.50 encapsulation dot1Q 50 ip address 192.168.5.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.60 encapsulation dot1Q 60 ip address 192.168.6.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.70 encapsulation dot1Q 70 ip address 192.168.7.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.80 encapsulation dot1Q 80 ip address 192.168.8.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.90 encapsulation dot1Q 90 ip address 192.168.9.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.100 encapsulation dot1Q 100 ip address 192.168.100.1 255.255.255.0 exit interface gigabitEthernet 0/0/1.10 encapsulation dot1Q 10 ip address 192.168.1.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.20 encapsulation dot1Q 20 ip address 192.168.2.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.30 encapsulation dot1Q 30 ip address 192.168.3.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.40 encapsulation dot1Q 40 ip address 192.168.4.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.50 encapsulation dot1Q 50 ip address 192.168.5.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.60 encapsulation dot1Q 60 ip address 192.168.6.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.70 encapsulation dot1Q 70 ip address 192.168.7.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.80 encapsulation dot1Q 80 ip address 192.168.8.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.90 encapsulation dot1Q 90 ip address 192.168.9.2 255.255.255.0 exit interface gigabitEthernet 0/0/1.100 encapsulation dot1Q 100 ip address 192.168.100.2 255.255.255.0 exit | BRANCH - CAMPUS interface gigabitEthernet 0/0/0.90 encapsulation dot1Q 90 ip address 192.168.9.1 255.255.255.0 exit interface gigabitEthernet 0/0/0.100 encapsulation dot1Q 100 ip address 192.168.10.1 255.255.255.0 exit interface gigabitEthernet 0/0/1.90 encapsulation dot1Q 90 ip address 192.168.9.1 255.255.255.0 exit interface gigabitEthernet 0/0/1.100 encapsulation dot1Q 100 ip address 192.168.10.1 255.255.255.0 exit |
| --- | --- |

# Configuring DHCP Server (Router as the DHCP Server).

| MAIN-MULTILAYER-SWITCH enable configure terminal service dhcp ip dhcp pool Admin-pool network 192.168.1.0 255.255.255.0 default-router 192.168.1.1 dns-server 192.168.1.1 domain-name admin.com exit ip dhcp pool HR-pool network 192.168.2.0 255.255.255.0 default-router 192.168.2.1 dns-server 192.168.2.1 domain-name hr.com exit ip dhcp pool Finance-pool network 192.168.3.0 255.255.255.0 default-router 192.168.3.1 dns-server 192.168.3.1 domain-name finance.com exit ip dhcp pool Business-pool network 192.168.4.0 255.255.255.0 default-router 192.168.4.1 dns-server 192.168.4.1 domain-name business.com exit ip dhcp pool E-C-pool network 192.168.5.0 255.255.255.0 default-router 192.168.5.1 dns-server 192.168.5.1 domain-name eandc.com exit ip dhcp pool A-D-pool network 192.168.6.0 255.255.255.0 default-router 192.168.6.1 dns-server 192.168.6.1 domain-name aand.com exit ip dhcp pool StdLab-pool network 192.168.7.0 255.255.255.0 default-router 192.168.7.1 dns-server 192.168.7.1 domain-name stdlab.com exit ip dhcp pool Server-pool network 192.168.8.0 255.255.255.0 default-router 192.168.8.1 dns-server 192.168.8.1 domain-name server.com exit | BRANCH - CAMPUS enable configure terminal service dhcp ip dhcp pool Staff-pool network 192.168.9.0 255.255.255.0 default-router 192.168.9.1 dns-server 192.168.9.1 domain-name staff.com exit ip dhcp pool Studentlab-pool network 192.168.10.0 255.255.255.0 default-router 192.168.10.1 dns-server 192.168.10.1 domain-name studentlab.com exit do wr |
| --- | --- |

# Configuring RIPv2 as the routing protocol.

| MAIN-MULTILAYER-SWITCH enable configure terminal router rip version 2 no auto-summary network 10.10.10.0 network 10.10.10.4 network 192.168.1.0 network 192.168.2.0 network 192.168.3.0 network 192.168.4.0 network 192.168.5.0 network 192.168.6.0 network 192.168.7.0 network 192.168.8.0 do wr | BRANCH CAMPUS enable configure terminal router rip version 2 no auto-summary network 10.10.10.0 network 10.10.10.8 network 192.168.9.0 network 192.168.10.0 exit do wr |
| --- | --- |
| MAIN-MULTILAYER-SWITCH enable configure terminal router rip version 2 no auto-summary network 20.0.0.0 network 10.10.10.4 network 10.10.10.8 exit do wr |  |

# Configuring SSH for secure Remote access.

| ADMIN DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name admin.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr | HR DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name hr.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr |
| --- | --- |
| FINANCE DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name finance.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr | BUSINESS DEPATMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name business.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr |
| E&C DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name eandc.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr | E & D DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name eandd.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr |
| STD-LAP DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name stdlab.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr | SERVER DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name server.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr |
| STAFF DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name staff.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr | STD DEPARTMENT enable configure terminal username admin privilege 1 secret admin username cisco privilege 5 secret cisco username subash privilege 15 secret subash ip domain-name std.com crypto key generate rsa 1024 ip ssh version 2 line con 0 login local exit line vty 0 4 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 exit wr |

# Configuring switchport security or Port-Security on the switches.

| interface range fastEthernet 0/1-23, gigabitEthernet 0/1-2 switchport port-security switchport port-security maximum 1 switchport port-security violation shutdown switchport port-security mac-address sticky exit | interface range gigabitEthernet 1/0/1-23 switchport port-security switchport port-security maximum 1 switchport port-security violation shutdown switchport port-security mac-address sticky exit |
| --- | --- |