Small Office Home Office Network -SOHO

CISCO Project 1

SUBASH SUBEDI

# Project #2 Case Study and Requirements

Design a network in Cisco Packet Tracer of a Small Office Small Office Home Office Network -SOHO

XYZ company is a fast-growing company in Eastern Australia with more than 2 million customers globally. The company deals with selling and buying of food items, which are basically operated from the headquarters. The company is intending to open a branch near the local village Bonalbo. Thus, the company requires young IT graduates to design the network for the branch. The network is intended to operate separately from the HQ network. Being a small network, the company has the following requirements during implementation;

One router and one switch to be used (all CISCO products).

3 departments (Admin/IT, Finance/HR and Customer service/Reception).

Each department is required to be in different VIANS.

Each department is required to have a wireless network for the users.

Host devices in the network are required to obtain IPv4 address automatically.

Devices in all the departments are required to communicate with each other.

Assume the ISP gave out a base network of 192.168.1.0, you as the young network engineer who has been hired, design and implement a network considering the above requirements.

Table of Contents

1.	Creating a Simple Network using a Router and Access Layer Switch.	3

2.	Connecting Networking devices with Correct cabling.	3

3.	Creating VLANs and assigning ports VLAN numbers.	3

4.	Subnetting and IP Addressing.	4

5.	Configuring Inter-VLAN Routing (Router on a stick).	5

6.	Configuring DHCP Server (Router as the DHCP Server).	5

7.	Configuring WLAN or wireless network (Cisco Access Point).	6

8.	Host Device Configurations.	6

9.	Test and Verifying Network Communication.	7

Technologies Implemented

## Creating a Simple Network using a Router and Access Layer Switch.

Figure 1

## Connecting Networking devices with Correct cabling.

Copper straight cable

## Creating VLANs and assigning ports VLAN numbers.

Creating VLAN on Switch

S1(config)#vlan 10 S1(config-vlan)#name ADMIN/IT S1(config-vlan)#exit

S1(config)#vlan 20 S1(config-vlan)#name FINANCE/HR S1(config-vlan)#exit

S1(config)#vlan 30 S1(config-vlan)#name CR/RECEPTION S1(config-vlan)#exit

assigning VLAN on interface on switch

S1(config)#interface range fastEthernet 0/1-4 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vla10 S1(config-if-range)#exit

S1(config)#interface range fastEthernet 0/5-8 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 20 S1(config-if-range)#exit

S1(config)#interface range fastEthernet 0/9-12 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 30 S1(config-if-range)#exit

S1(config)#interface gigabitEthernet 0/1 S1(config-if)#switchport mode trunk S1(config-if)#no shutdown S1(config-if)#exit

## Subnetting and IP Addressing.

subnet: 255.255.255.192 (/26)

Block Size = 64

| DEPARTMENT | NETWORK ID | GATEWAYS / STARTING IP | LAST IP | BROADCAST ID | SUBNET MASK |
| --- | --- | --- | --- | --- | --- |
| 1ST | 192.168.1.0/26 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 | 255.255.255.192 |
| 2ND | 192.168.1.64/26 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 | 255.255.255.192 |
| 3RD | 192.168.1.128/26 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 | 255.255.255.192 |

## Configuring Inter-VLAN Routing (Router on a stick).

Core_Router(config)#interface gigabitEthernet 0/0/0.10 Core_Router(config-subif)#encapsulation dot1Q 10 Core_Router(config-subif)#ip address 192.168.1.1 255.255.255.192 //on ip address set gateways Core_Router(config-subif)#exit

Core_Router(config)#interface gigabitEthernet 0/0/0.20 Core_Router(config-subif)#encapsulation dot1Q 20 Core_Router(config-subif)#ip address 192.168.1.65 255.255.255.192 //on ip address set gateways Core_Router(config-subif)#exit

Core_Router(config)#int gigabitEthernet 0/0/0.30 Core_Router(config-subif)#encapsulation dot1Q 30 Core_Router(config-subif)#ip address 192.168.1.129 255.255.255.192 //on ip address set gateways Core_Router(config-subif)#exit

## Configuring DHCP Server (Router as the DHCP Server).

Core_Router(config)#service dhcp

Core_Router(config)#ip dhcp pool Admin-pool Core_Router(dhcp-config)#network 192.168.1.0 255.255.255.192 Core_Router(dhcp-config)#default-router 192.168.1.1 Core_Router(dhcp-config)#dns-server 192.168.1.1 Core_Router(dhcp-config)#domain-name Admin.com Core_Router(dhcp-config)#exit explain each cmd in deep details

Core_Router(config)#ip dhcp pool Finance-Pool Core_Router(dhcp-config)#network 192.168.1.64 255.255.255.192 Core_Router(dhcp-config)#default-router 192.168.1.65 Core_Router(dhcp-config)#dns-server 192.168.1.65 Core_Router(dhcp-config)#domain-name finance.com Core_Router(dhcp-config)#exit

Core_Router(config)#ip dhcp pool CS-Pool Core_Router(dhcp-config)#network 192.168.1.128 255.255.255.192 Core_Router(dhcp-config)#default-router 192.168.1.129 Core_Router(dhcp-config)#dns-server 192.168.1.129 Core_Router(dhcp-config)#domain-name cs.com Core_Router(dhcp-config)#exit

If you are configuring a Cisco router or switch, you should use:

Router#show ip dhcp binding

This command will display the list of devices that have received an IP from the DHCP server.

## Configuring WLAN or wireless network (Cisco Access Point).

All the Wireless AP will automatically learn the IP address From Core Router.

## Host Device Configurations.

Select the device and on that you find the Desktop on that you will find the Command Prompt types cmd ipconfig /renew

## Test and Verifying Network Communication.