Subash Subedi

Design and Implementation of a Hotel System Network Design

Table of Contents

Design and Implementation of an Hotel System Network Design (Project #3)	2

1.	Creating a network topology using Cisco Packet Tracer.	3

2.	Connecting Networking devices with Correct cabling.	3

3.	Creating VLANs and assigning ports VLAN numbers.	4

4.	Subnetting and IP Addressing.	7

5.	Configuring Inter-VLAN Routing (Router on a stick).	8

6.	Configuring DHCP Server (Router as the DHCP Server).	10

7.	Configuring SSH for secure Remote access.	12

8.	Configuring switchport security or Port-Security on the switches.	14

1.	Configuring WLAN or wireless network (Cisco Access Point).	Error! Bookmark not defined.

9.	Configure OSPF inn cisco router	15

10.	Test and Verifying Network Communication.	16

# Design and Implementation of a Hotel System Network Design (Project #3)

Project #3 Case Study and Requirements

As a part of your end-of-year networking project, you are required to design and implement the Vic Modern Hotel network. The hotel has three floors; on the first floor there three departments (Reception, store and Logistics), on the second floor there are three departments (Finance, HR and Sales/Marketing), while the third floor hosts the IT and Admin. Therefore, the following are part of the considerations during the design and implementation;

- There should be three routers connecting each floor (all placed in the server room in the IT department).

- All routers should be connected to each other using serial DCE cables.

- The network between the routers should be 10.10.10.0/30,10.10.10.4/30 and 10.10.10.8/30.

- Each floor is expected to have one switch (placed on the respective floor).

- Each floor is expected to have WIFI networks connected to laptops and phones.

- Each department is expected to have a printer.

- Each department is expected to be in a different VLAN with the following details.

1st Floor.

- Reception- VLAN 80, Network of 192.168.8.0/24

- Store- VLAN 70, Network of 192.168.7.0/24

- Logistics- VLAN 60, Network of 192.168.6.0/24

2nd Floor.

- Finance- VLAN 50, Network of 192.168.5.0/24

- HR- VLAN 40, Network of 192.168.4.0/24

- Sales- VLAN 30, Network of 192.168.3.0/24

3rd Floor.

- Admin- VLAN 20, Network of 192.168.2.0/24

- IT- VLAN 10, Network of 192.168.1.0/24

- Use OSPF as the routing protocol to advertise routes.

- All devices in the network are expected to obtain IP address dynamically with their respective router configured as the DHCP server.

- All the devices in the network are expected to communicate with each other.

- Configure SSH in all the routers for remote login.

- In the IT department, add PC called Test-PC to port fa0/1 and use it to test remote login.

- Configure port security to IT-dept switch to allow only Test-PC to access port fa0/1 (use sticky method to obtain mac-address with violation mode of shutdown.)

Technologies Implemented

## Creating a network topology using Cisco Packet Tracer.

Figure 1

## Connecting Networking devices with Correct cabling.

Copper straight cable

Serial cable

## Creating VLANs and assigning ports VLAN numbers.

CREATE FIRST FLOOR VLAN

1st-floor-switch(config)#vlan 80 1st-floor-switch(config-vlan)#name Reception_Department 1st-floor-switch(config-vlan)#exit

1st-floor-switch(config)#vlan 70 1st-floor-switch(config-vlan)#name Stored_Department 1st-floor-switch(config-vlan)#exit

1st-floor-switch(config)#vlan 60 1st-floor-switch(config-vlan)#name Logistics_Department 1st-floor-switch(config-vlan)#exit

CREATE SECOND FLOOR VLAN

2nd-floor-switch(config)#vlan 30 2nd-floor-switch(config-vlan)#name Sales_Department 2nd-floor-switch(config-vlan)#exit

2nd-floor-switch(config)#vlan 40 2nd-floor-switch(config-vlan)#name Hr_Department 2nd-floor-switch(config-vlan)#exit

2nd-floor-switch(config)#vlan 50 2nd-floor-switch(config-vlan)#name Finance_Department 2nd-floor-switch(config-vlan)#exit

CREATE THIRD FLOOR VLAN

3rd-floor-switch(config)#vlan 10 3rd-floor-switch(config-vlan)#name IT_Department 3rd-floor-switch(config-vlan)#exit

3rd-floor-switch(config)#vlan 20 3rd-floor-switch(config-vlan)#name Admin_Department 3rd-floor-switch(config-vlan)#exit

ASSIGNING PORTS VLAN

1ST FLOOR

1st-floor-switch(config)#interface range fastEthernet 0/1-8 1st-floor-switch(config-if-range)#description $THIS LAN IS FOR RECEPTION VLAN-80$ 1st-floor-switch(config-if-range)#switchport mode access 1st-floor-switch(config-if-range)#switchport access vlan 80 1st-floor-switch(config-if-range)#exit

1st-floor-switch(config)#interface range fastEthernet 0/9-16 1st-floor-switch(config-if-range)#description $THIS LAN IS FOR STORED VLAN-70$ 1st-floor-switch(config-if-range)#switchport mode access 1st-floor-switch(config-if-range)#switchport access vlan 70 1st-floor-switch(config-if-range)#exit

1st-floor-switch(config)#interface range fastEthernet 0/17-24 1st-floor-switch(config-if-range)#description $THIS LAN IS FOR LOGISTICS VLAN-60$ 1st-floor-switch(config-if-range)#switchport mode access 1st-floor-switch(config-if-range)#switchport access vlan 60 1st-floor-switch(config-if-range)#exit

TRUNK INTERFACE 1st-floor-switch(config)#interface range gigabitEthernet 0/1-2 1st-floor-switch(config-if-range)#description $THIS LAN IS FOR TRUNK$ 1st-floor-switch(config-if-range)#switchport mode trunk 1st-floor-switch(config-if-range)#exit

2ND FLOOR

2nd-floor-switch(config)#interface range fastEthernet 0/1-8 2nd-floor-switch(config-if-range)#description $THIS LAN IS FOR SALES VLAN-30$ 2nd-floor-switch(config-if-range)#switchport mode access 2nd-floor-switch(config-if-range)#switchport access vlan 30 2nd-floor-switch(config-if-range)#exit

2nd-floor-switch(config)#interface range fastEthernet 0/9-16 2nd-floor-switch(config-if-range)#description $THIS LAN IS FOR HR VLAN-40$ 2nd-floor-switch(config-if-range)#switchport mode access 2nd-floor-switch(config-if-range)#switchport access vlan 40 2nd-floor-switch(config-if-range)#exit

2nd-floor-switch(config)#interface range fastEthernet 0/17-24 2nd-floor-switch(config-if-range)#description $THIS LAN IS FOR FINANCE VLAN-50 $ 2nd-floor-switch(config-if-range)#switchport mode access 2nd-floor-switch(config-if-range)#switchport access vlan 50 2nd-floor-switch(config-if-range)#exit

TRUNK INTERFACE 2nd-floor-switch(config)#interface range gigabitEthernet 0/1-2 2nd-floor-switch(config-if-range)#description $THIS LAN IS FOR TRUNK$ 2nd-floor-switch(config-if-range)#switchport mode trunk 2nd-floor-switch(config-if-range)#exit

3RD FLOOR

3rd-floor-switch(config)#interface ran fa 0/1-12 3rd-floor-switch(config-if-range)#description $THIS LAN IS FOR IT VLAN-10$ 3rd-floor-switch(config-if-range)#switchport mode access 3rd-floor-switch(config-if-range)#switchport access vlan 10 3rd-floor-switch(config-if-range)#exit 3rd-floor-switch(config)#do wr

3rd-floor-switch(config)#interface range fastEthernet 0/13-24 3rd-floor-switch(config-if-range)#description $THIS LAN IS FOR ADMIN VLAN-20$ 3rd-floor-switch(config-if-range)#switchport mode access 3rd-floor-switch(config-if-range)#exit

TRUNK PORT 3rd-floor-switch(config)#interface range gigabitEthernet 0/1-2 3rd-floor-switch(config-if-range)#description $THIS LAN IS TRUNK $ 3rd-floor-switch(config-if-range)#switchport mode trunk 3rd-floor-switch(config-if-range)#exit

## Subnetting and IP Addressing.

| Floor | DEPARTMENT | NETWORK ID | GATEWAYS / STARTING IP | LAST IP | BROADCAST ID | SUBNET MASK |
| --- | --- | --- | --- | --- | --- | --- |
| 3rd Floor | IT Department | 192.168.1.0/24 | 192.168.1.1 | 192.168.1.254 | 192.168.1.255 | 255.255.255.0 |
|  | Admin Department | 192.168.2.0/24 | 192.168.2.1 | 192.168.2.254 | 192.168.2.255 | 255.255.255.0 |
| 2nd Floor | Sales Department | 192.168.3.0/24 | 192.168.3.1 | 192.168.3.254 | 192.168.3.255 | 255.255.255.0 |
|  | HR Department | 192.168.4.0/24 | 192.168.4.1 | 192.168.4.254 | 192.168.4.255 | 255.255.255.0 |
|  | Finance Department | 192.168.5.0/24 | 192.168.5.1 | 192.168.5.254 | 192.168.5.255 | 255.255.255.0 |
| 1st Floor | Logistics Department | 192.168.6.0/24 | 192.168.6.1 | 192.168.6.254 | 192.168.6.255 | 255.255.255.0 |
|  | Store Department | 192.168.7.0/24 | 192.168.7.1 | 192.168.7.254 | 192.168.7.255 | 255.255.255.0 |
|  | Reception Department | 192.168.8.0/24 | 192.168.8.1 | 192.168.8.254 | 192.168.8.255 | 255.255.255.0 |

## Configuring Inter-VLAN Routing (Router on a stick).

FRIST FLOOR ROUTER

1st-Floor-Router(config)#interface gigabitEthernet 0/0/0.60 1st-Floor-Router(config-subif)#encapsulation dot1Q 60 1st-Floor-Router(config-subif)#ip address 192.168.6.1 255.255.255.0 1st-Floor-Router(config-subif)#no shutdown 1st-Floor-Router(config-subif)#exit

1st-Floor-Router(config)#interface gigabitEthernet 0/0/0.70 1st-Floor-Router(config-subif)#encapsulation dot1Q 70 1st-Floor-Router(config-subif)#ip address 192.168.7.1 255.255.255.0 1st-Floor-Router(config-subif)#no shutdown

1st-Floor-Router(config)#interface gigabitEthernet 0/0/0.80 1st-Floor-Router(config-subif)#encapsulation dot1Q 80 1st-Floor-Router(config-subif)#ip add 192.168.8.1 255.255.255.0 1st-Floor-Router(config-subif)#no shutdown 1st-Floor-Router(config-subif)#exit

SECOND FLOOR ROUTER

2nd-Floor-Router(config)#interface gigabitEthernet 0/0/0.30 2nd-Floor-Router(config-subif)#encapsulation dot1Q 30 2nd-Floor-Router(config-subif)#ip address 192.168.3.1 255.255.255.0 2nd-Floor-Router(config-subif)#no shutdown 2nd-Floor-Router(config-subif)#exit

2nd-Floor-Router(config)#interface gigabitEthernet 0/0/0.40 2nd-Floor-Router(config-subif)#encapsulation dot1Q 40 2nd-Floor-Router(config-subif)#ip address 192.168.4.1 255.255.255.0 2nd-Floor-Router(config-subif)#no shutdown 2nd-Floor-Router(config-subif)#exit

2nd-Floor-Router(config)#interface gigabitEthernet 0/0/0.50 2nd-Floor-Router(config-subif)#encapsulation dot1Q 50 2nd-Floor-Router(config-subif)#ip address 192.168.5.1 255.255.255.0 2nd-Floor-Router(config-subif)#no shutdown 2nd-Floor-Router(config-subif)#exit

THIRD FLOOR ROUTER

2nd-Floor-Router(config)#interface gigabitEthernet 0/0/0.10 2nd-Floor-Router(config-subif)#encapsulation dot1Q 10 2nd-Floor-Router(config-subif)#ip address 192.168.1.1 255.255.255.0 2nd-Floor-Router(config-subif)#no shutdown 2nd-Floor-Router(config-subif)#exit

2nd-Floor-Router(config)#interface gigabitEthernet 0/0/0.20 2nd-Floor-Router(config-subif)#encapsulation dot1Q 20 2nd-Floor-Router(config-subif)#ip address 192.168.2.1 255.255.255.0 2nd-Floor-Router(config-subif)#no shutdown 2nd-Floor-Router(config-subif)#exit

## Configuring DHCP Server (Router as the DHCP Server).

FRIST FLOOR ROUTER

1st-Floor-Router(config)#service dhcp

1st-Floor-Router(config)#ip dhcp pool Logistic-Pool 1st-Floor-Router(dhcp-config)#network 192.168.6.0 255.255.255.0 1st-Floor-Router(dhcp-config)#dns-server 8.8.8.8 1st-Floor-Router(dhcp-config)#default-router 192.168.6.1 1st-Floor-Router(dhcp-config)#exit

1st-Floor-Router(config)#ip dhcp pool Store-pool 1st-Floor-Router(dhcp-config)#network 192.168.7.0 255.255.255.0 1st-Floor-Router(dhcp-config)#default-router 192.168.7.1 1st-Floor-Router(dhcp-config)#dns-server 8.8.8.8 1st-Floor-Router(dhcp-config)#domain-name store.com 1st-Floor-Router(dhcp-config)#exit

1st-Floor-Router(config)#ip dhcp pool Reception-pool 1st-Floor-Router(dhcp-config)#network 192.168.8.0 255.255.255.0 1st-Floor-Router(dhcp-config)#default-router 192.168.1.1 1st-Floor-Router(dhcp-config)#dns-server 8.8.8.8 1st-Floor-Router(dhcp-config)#exit

SECOND FLOOR ROUTER

2nd-Floor-Router(config)#service dhcp

2nd-Floor-Router(config)#ip dhcp pool Sales-pool 2nd-Floor-Router(dhcp-config)#network 192.168.3.0 255.255.255.0 2nd-Floor-Router(dhcp-config)#default-router 192.168.3.1 2nd-Floor-Router(dhcp-config)#dns-server 8.8.8.8 2nd-Floor-Router(dhcp-config)#domain-name sales.com 2nd-Floor-Router(dhcp-config)#exit

2nd-Floor-Router(config)#ip dhcp pool HR-Pool 2nd-Floor-Router(dhcp-config)#network 192.168.4.0 255.255.255.0 2nd-Floor-Router(dhcp-config)#default-router 192.168.4.1 2nd-Floor-Router(dhcp-config)#dns-server 8.8.8.8 2nd-Floor-Router(dhcp-config)#domain-name hr.com 2nd-Floor-Router(dhcp-config)#exit

2nd-Floor-Router(config)#ip dhcp pool Finance-Pool 2nd-Floor-Router(dhcp-config)#network 192.168.5.0 255.255.255.0 2nd-Floor-Router(dhcp-config)#default-router 192.168.5.1 2nd-Floor-Router(dhcp-config)#dns-server 8.8.8.8 2nd-Floor-Router(dhcp-config)#domain-name finance.com 2nd-Floor-Router(dhcp-config)#exit

THIRD FLOOR ROUTER

3rd-Floor-Router(config)#service dhcp

3rd-Floor-Router(config)#ip dhcp pool IT-Pool 3rd-Floor-Router(dhcp-config)#network 192.168.1.0 255.255.255.0 3rd-Floor-Router(dhcp-config)#default-router 192.168.1.1 3rd-Floor-Router(dhcp-config)#dns-server 8.8.8.8 3rd-Floor-Router(dhcp-config)#domain-name it.com 3rd-Floor-Router(dhcp-config)#exit

3rd-Floor-Router(config)#ip dhcp pool Admin-pool 3rd-Floor-Router(dhcp-config)#network 192.168.2.0 255.255.255.0 3rd-Floor-Router(dhcp-config)#default-router 192.168.2.1 3rd-Floor-Router(dhcp-config)#dns-server 8.8.8.8 3rd-Floor-Router(dhcp-config)#domain-name admin.com 3rd-Floor-Router(dhcp-config)#exit

## Configuring SSH for secure Remote access.

First Floor

1st-Floor-Router(config)#ip domain-name 1st-floor-router.com

1st-Floor-Router(config)#crypto key generate rsa

How many bits in the modulus [512]: 2048

1st-Floor-Router(config)#ip ssh version 2

1st-Floor-Router(config)#username admin privilege 5 secret admin

1st-Floor-Router(config)#username cisco privilege 5 secret cisco

1st-Floor-Router(config)#line vty 0 4

1st-Floor-Router(config-line)#transport input ssh

1st-Floor-Router(config-line)#login local

1st-Floor-Router(config-line)#exit

Second Floor

2nd-Floor-Router(config)#ip domain-name 2nd-floor-router.com

2nd-Floor-Router(config)#crypto key generate rsa

How many bits in the modulus [512]: 2048

2nd-Floor-Router(config)#ip ssh version 2

2nd-Floor-Router(config)#username admin privilege 5 secret admin

2nd-Floor-Router(config)#username cisco privilege 5 secret cisco

2nd-Floor-Router(config)#line vty 0 4

2nd-Floor-Router(config-line)#transport input ssh

2nd-Floor-Router(config-line)#login local

2nd-Floor-Router(config-line)#exit

Third Floor

3rd-Floor-Router(config)#ip domain-name 3rd-floor-router.com

3rd-Floor-Router(config)#crypto key generate rsa

How many bits in the modulus [512]: 2048

3rd-Floor-Router(config)#ip ssh version 2

3rd-Floor-Router(config)#username admin privilege 5 secret admin

3rd-Floor-Router(config)#username cisco privilege 5 secret cisco

3rd-Floor-Router(config)#line vty 0 4

3rd-Floor-Router(config-line)#transport input ssh

3rd-Floor-Router(config-line)#login local

3rd-Floor-Router(config-line)#exit

## Configuring switchport security or Port-Security on the switches.

First Floor Switch

1st-floor-switch(config)#interface range fastEthernet 0/1-24

1st-floor-switch(config-if-range)#switchport port-security

1st-floor-switch(config-if-range)#switchport port-security maximum 1

1st-floor-switch(config-if-range)#switchport port-security violation shutdown

1st-floor-switch(config-if-range)#switchport port-security mac-address sticky

1st-floor-switch(config-if-range)#end

Second Floor Switch

2nd-floor-switch(config)#interface range fastEthernet 0/1-24

2nd-floor-switch(config-if-range)#switchport port-security

2nd-floor-switch(config-if-range)#switchport port-security maximum 1

2nd-floor-switch(config-if-range)#switchport port-security violation shutdown

2nd-floor-switch(config-if-range)#switchport port-security mac-address sticky

2nd-floor-switch(config-if-range)#end

Third Floor

3rd-floor-switch(config)#interface range fastEthernet 0/1-24

3rd-floor-switch(config-if-range)#switchport port-security

3rd-floor-switch(config-if-range)#switchport port-security maximum 1

3rd-floor-switch(config-if-range)#switchport port-security violation shutdown

3rd-floor-switch(config-if-range)#switchport port-security mac-address sticky

3rd-floor-switch(config-if-range)#end

## Configure OSPF inn cisco router

First Floor

1st-Floor-Router(config)#router ospf 1

1st-Floor-Router(config-router)#network 10.10.10.8 0.0.0.3 area 0

1st-Floor-Router(config-router)#network 10.10.10.8 0.0.0.3 area 0

1st-Floor-Router(config-router)#network 192.168.6.0 0.0.0.255 area 0

1st-Floor-Router(config-router)#network 192.168.7.0 0.0.0.255 area 0

1st-Floor-Router(config-router)#network 192.168.8.0 0.0.0.255 area 0

1st-Floor-Router(config-router)#exit

Second Floor

2nd-Floor-Router(config)#router ospf 1

2nd-Floor-Router(config-router)#network 10.10.10.4 0.0.0.3 area 0

2nd-Floor-Router(config-router)#network 10.10.10.8 0.0.0.3 area 0

2nd-Floor-Router(config-router)#network 192.168.3.0 0.0.0.255 area 0

2nd-Floor-Router(config-router)#network 192.168.4.0 0.0.0.255 area 0

2nd-Floor-Router(config-router)#network 192.168.5.0 0.0.0.255 area 0

2nd-Floor-Router(config-router)#exit

Third Floor

3rd-Floor-Router(config)#router ospf 1

3rd-Floor-Router(config-router)#network 10.10.10.4 0.0.0.3 area 0

3rd-Floor-Router(config-router)#network 10.10.10.0 0.0.0.3 area 0

3rd-Floor-Router(config-router)#network 192.168.1.0 0.0.0.255 area 0

3rd-Floor-Router(config-router)#network 192.168.2.0 0.0.0.255 area 0

3rd-Floor-Router(config-router)#exit

## Test and Verifying Network Communication.

| Figure 2 | Figure 3 | Figure 4 |
| --- | --- | --- |