Subash Subedi

[Email address]

Design and Implementation of a Hospital System Network Design (Project #7)

CCNA

# Design and Implementation of a Hospital System Network Design (Project #7)

Melbourne Health Services is a well-established health provider in Australia, which offers health solutions and services to its clients. The institution operates in two locations within the same city, having the hospital headquarters 20km away from the branch hospital. Therefore, it has the following departments within its main headquarters Medical Lead Operation & Consultancy Services (MLOCS), Medical Emergency and Reporting (MER), Medical Records Management (MRM), Information Technology (IT), and Customer Service (CS). The hospital branch was designed to share the workload with the headquarters hence it contains the following departments: Nurses & Surgery Operations (NSO), Hospital Labs (HL), Human Resources (HR), Marketing (MK), and Finance (FIN). Each location is also expected to have a Guest/Waiting area (GWA) for patients or visitors.

So far, the network has been using third-party services to maintain its IT services. The senior management has decided to own their network infrastructure including Local Area Network (LAN), Wide Area Network (WAN), and a Server-Side site that is expected to be located separately at the headquarters and is connected to the HQ Router with an access switch. The server-side site will host the DHCP server, DNS Server, Web Server, and Email Server. The network is expected to be cost-effective and observes the information security rule of the CIA (Confidentiality, Integrity, and Availability).

The network is expected to have a hierarchical model with two already purchased Core routers (one at HQ and one Branch) each connecting two subscribed ISPs. Due to security requirements, it has been decided that all the departments will be on a separate network segment within the same local area network.

You have been hired as a network security engineer to design the network according to the requirements set by the senior management. You will consult an appropriate robust network design model to meet the design requirements. You will also implement Access Control Lists and Virtual Private Network (VPN) to enable secure communication considering security and network performance factors paramount to safeguarding Confidentiality, Integrity, and Availability of data and communication. The network security policy will comprehensively dictate the user's access to each site using Access Control List (ACL).

- Use Cisco Packet Tracer to design and implement the network solution.

- Use a hierarchical model providing redundancy in the network.

- Both HQ and Branch routers are expected to be connected using a serial connection.

- As mentioned earlier, for network cost-effectiveness, each site is expected to have one core router, two multilayer switches, and several access switches connecting each department.

- Each department is required to have a wireless network for the users.

- Every department in HQ is estimated to have around 60 users while in Branch it is estimated to be 30 users.

- Each department should be in a different VLAN and a different subnetwork.

- Provided a base network of 192.168.100.0 and carried out subnetting to allocate the correct number of IP addresses to each department.

- The company network is connected to the static, public IP addresses (Internet Protocol) 195.136.17.0/30, 195.136.17.4/30, 195.136.17.8/30, and 195.136.17.12/30 connected to the two Internet providers.

- Configure basic device settings such as hostnames, console password, enable password, banner messages, and disable IP domain lookup.

- Devices in all the departments are required to communicate with each other with the respective multilayer switch configured for inter-VLAN routing.

- The Multilayer switches are expected to carry out both routing and switching functionalities and thus will be assigned IP addresses.

- All devices in the network are expected to obtain an IP address dynamically from the dedicated DHCP servers located in the server room.

- Devices in the server room are to be allocated IP addresses statically.

- Use OSPF as the routing protocol to advertise routes both on the routers and multilayer switches.

- Configure default static routing to enable routers and multilayer switches to forward any traffic that does not match routing table entries. Use next-hop IP addresses.

- Configure SSH in all the routers and layer three switches for remote login.

- Configure port-security for the server site department switch to allow only one device to connect to a switch port, use sticky method to obtain mac-address and violation mode shutdown.

- Configure the extended ACL rule together with site-to-site VPN (IPSec VPN) to create a tunnel and encrypt communication between HQ and the Branch network.

- Configure PAT to use the respective outbound router interface IPv4 address and implement the necessary ACL rule.

- Test Communication, ensure everything configured is working as expected.

Table of Contents

Technologies Implemented	4

1.	Creating a network topology using Cisco Packet Tracer. Hierarchical Network Design.	4

2.	Connecting Networking devices with Correct cabling.	4

3.	Configuring Basic device settings.	4

4.	Creating VLANs and assigning ports VLAN numbers.	4

5.	Subnetting and IP Addressing.	4

6.	Configuring Inter-VLAN Routing on the Multilayer switches (Switch Virtual Interface).	4

7.	Configuring Dedicated DHCP Server device to provide dynamic IP allocation.	4

8.	Configuring SSH for secure Remote access.	4

9.	Configuring OSPF as the routing protocol.	4

10.	Configuring NAT Overload (Port Address Translation PAT).	4

11.	Configuring Site-to-Site IPsec VPN.	4

12.	Configuring standard and extended Access Control Lists ACL.	4

13.	Configuring switchport security or Port-Security on the switches.	4

14.	Configuring WLAN or wireless network (Cisco Access Point).	4

15.	Host Device Configurations.	4

16.	Configuring ISP routers.	4

17.	Test and Verifying Network Communication.	4

# Technologies Implemented

## Creating a network topology using Cisco Packet Tracer. Hierarchical Network Design.

## Connecting Networking devices with Correct cabling.

Copper Straight-Through

Copper Cross-Over

ISP Router Model: 2811

Core Router Model: ISR 4311

Multi-Layer Switch Model: 3650-24P

L2-Switch Model: 2960-24TT

Laptop-PT

Printer-PT

Access Point PT-AC

Tablet PC-PT

SmartPhone-PT

Server-PT

## Configuring Basic device settings.

| MAIN HO PRIMARY ROUTER enable configure terminal hostname HO-PRIMARY-ROUTER do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal hostname BRANCH-PRIMARY-ROUTER do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| --- | --- |
| MAIN HO SECONDARY ROUTER enable configure terminal hostname HO-SECONDARY-ROUTER do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal hostname BRANCH-SECONDARY-ROUTER do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |

| PRIMARY HO MULTILAYER SWITCH enable configure terminal hostname PRIMARY-HO-MULTILAYER-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | PRIMARY BRANCH MULTILAYER SWITCH enable configure terminal hostname PRIMARY-BRANCH-MULTILAYER-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| --- | --- |
| SECONDARY HO MULTILAYER SWITCH enable configure terminal hostname SECONDARY-HO-MULTILAYER-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | SECONDARY BRANCH MULTILAYER SWITCH enable configure terminal hostname SECONDARY-BRANCH-MULTILAYER-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |

| MLCOS SWITCH enable configure terminal hostname MLCOS-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | NSO SWITCH enable configure terminal hostname NSO-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| --- | --- |
| MER SWITCH enable configure terminal hostname MER-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | HL SWITCH enable configure terminal hostname HL-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| MRM SWITCH enable configure terminal hostname MRM-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | HR SWITCH enable configure terminal hostname HR-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| IT SWITCH enable configure terminal hostname IT-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | MK SWITCH enable configure terminal hostname MK-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| CS SWITCH enable configure terminal hostname CS-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | FIN ROOM SWITCH enable configure terminal hostname FIN-ROOM-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
| GWA SWITCH enable configure terminal hostname GWA-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr | BR-GWA SWITCH enable configure terminal hostname BR-GWA-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |
|  | Server Side SWITCH enable configure terminal hostname Server-Side-SWITCH do clock set 00:00:00 1 JANUARY 2025 banner motd $ ONLY AUTHORIZED ACCESS $ service password-encryption enable secret cisco username cisco secret cisco no ip domain lookup line console 0 motd-banner password cisco exec-timeout 5 login exit do wr |

## Creating VLANs and assigning ports VLAN numbers.

| PRIMARY & SECONDARY HO MULTILAYER SWITCH enable configure terminal vlan 999 name NATIVE-VLAN exit vlan 210 name MLOCS-DEPARTMENT exit interface GigabitEthernet 1/0/1 description **THIS IS TRUNK INTERFACE OF Medical Lead Operation & Consultancy Services (MLOCS) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 210 switchport nonegotiate exit vlan 240 name IT-DEPARTMENT exit interface GigabitEthernet 1/0/2 description **THIS IS TRUNK INTERFACE OF Information Technology (IT) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 240 switchport nonegotiate exit vlan 260 name GWA-DEPARTMENT exit interface GigabitEthernet 1/0/3 description **THIS IS TRUNK INTERFACE OF Guest/Waiting Area (GWA) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 260 switchport nonegotiate exit vlan 220 name MER-DEPARTMENT exit interface GigabitEthernet 1/0/4 description **THIS IS TRUNK INTERFACE OF Medical Emergency and Reporting (MER) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 220 switchport nonegotiate exit vlan 230 name MRM-DEPARTMENT exit interface GigabitEthernet 1/0/5 description **THIS IS TRUNK INTERFACE OF Medical Records Management (MRM) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 230 switchport nonegotiate exit vlan 250 name CS-DEPARTMENT exit interface GigabitEthernet 1/0/6 description **THIS IS TRUNK INTERFACE OF Customer Service (CS) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 250 switchport nonegotiate exit do wr | PRIMARY & SECONDARY BRANCH MULTILAYER SWITCH enable configure terminal vlan 999 name NATIVE-VLAN exit vlan 270 name NSO-DEPARTMENT exit interface GigabitEthernet 1/0/1 description **THIS IS TRUNK INTERFACE OF Nurses & Surgery Operations (NSO) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 270 switchport nonegotiate exit vlan 280 name HL-DEPARTMENT exit interface GigabitEthernet 1/0/2 description **THIS IS TRUNK INTERFACE OF Hospital Labs (HL) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 280 switchport nonegotiate exit vlan 290 name HR-DEPARTMENT exit interface GigabitEthernet 1/0/3 description **THIS IS TRUNK INTERFACE OF Human Resources (HR) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 290 switchport nonegotiate exit vlan 300 name MK-DEPARTMENT exit interface GigabitEthernet 1/0/4 description **THIS IS TRUNK INTERFACE OF Marketing (MK) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 300 switchport nonegotiate exit vlan 310 name FIN-DEPARTMENT exit interface GigabitEthernet 1/0/5 description **THIS IS TRUNK INTERFACE OF Finance (FIN) DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 310 switchport nonegotiate exit vlan 320 name BR-GWA-DEPARTMENT exit interface GigabitEthernet 1/0/6 description **THIS IS TRUNK INTERFACE OF BR-GWA DEPARTMENT ** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 320 switchport nonegotiate exit do wr |
| --- | --- |

| MLCOS SWITCH enable configure terminal vlan 210 name MLOCS-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR MLOCS DEPARTMENT** switchport mode access switchport access vlan 210 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 210 switchport nonegotiate exit interface vlan 210 description **MLOCS DEPARTMENT VLAN INTERFACE** ip address 172.16.200.62 255.255.255.192 no shutdown exit ip default-gateway 172.16.200.1 do wr | NSO SWITCH enable configure terminal vlan 270 name NSO-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Nurses & Surgery Operations (NSO) DEPARTMENT** switchport mode access switchport access vlan 270 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 270 switchport nonegotiate exit interface vlan 270 description **NSO DEPARTMENT VLAN INTERFACE** ip address 172.16.21.190 255.255.255.192 no shutdown exit ip default-gateway 172.16.21.129 do wr |
| --- | --- |
| MER SWITCH enable configure terminal vlan 220 name MER-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR MER DEPARTMENT** switchport mode access switchport access vlan 220 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 220 switchport nonegotiate exit interface vlan 220 description **MER DEPARTMENT VLAN INTERFACE** ip address 172.16.20.126 255.255.255.192 no shutdown exit ip default-gateway 172.16.20.65 do wr | HL SWITCH enable configure terminal vlan 280 name HL-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Hospital Labs (HL) DEPARTMENT** switchport mode access switchport access vlan 280 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 280 switchport nonegotiate exit interface vlan 280 description **HL DEPARTMENT VLAN INTERFACE** ip address 172.16.21.254 255.255.255.192 no shutdown exit ip default-gateway 172.16.21.193 do wr |
| MRM SWITCH enable configure terminal vlan 230 name MRM-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR MRM DEPARTMENT** switchport mode access switchport access vlan 230 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 230 switchport nonegotiate exit interface vlan 230 description **MRM DEPARTMENT VLAN INTERFACE** ip address 172.16.20.190 255.255.255.192 no shutdown exit ip default-gateway 172.16.20.129 do wr | HR SWITCH enable configure terminal vlan 290 name HR-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Human Resources (HR) DEPARTMENT** switchport mode access switchport access vlan 290 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 290 switchport nonegotiate exit interface vlan 290 description **HR DEPARTMENT VLAN INTERFACE** ip address 172.16.22.62 255.255.255.192 no shutdown exit ip default-gateway 172.16.22.1 do wr |
| IT SWITCH enable configure terminal vlan 240 name IT-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR IT DEPARTMENT** switchport mode access switchport access vlan 240 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 240 switchport nonegotiate exit interface vlan 240 description **IT DEPARTMENT VLAN INTERFACE** ip address 172.16.20.254 255.255.255.192 no shutdown exit ip default-gateway 172.16.20.193 do wr | MK SWITCH enable configure terminal vlan 300 name MK-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Marketing (MK) DEPARTMENT** switchport mode access switchport access vlan 300 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 300 switchport nonegotiate exit interface vlan 300 description **MK DEPARTMENT VLAN INTERFACE** ip address 172.16.22.126 255.255.255.192 no shutdown exit ip default-gateway 172.16.22.65 do wr |
| CS SWITCH enable configure terminal vlan 250 name CS-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR CS DEPARTMENT** switchport mode access switchport access vlan 250 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 250 switchport nonegotiate exit interface vlan 250 description **CS DEPARTMENT VLAN INTERFACE** ip address 172.16.21.62 255.255.255.192 no shutdown exit ip default-gateway 172.16.21.1 do wr | FIN ROOM SWITCH enable configure terminal vlan 310 name FIN-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Finance (FIN) DEPARTMENT** switchport mode access switchport access vlan 310 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 310 switchport nonegotiate exit interface vlan 310 description **FIN DEPARTMENT VLAN INTERFACE** ip address 172.16.22.190 255.255.255.192 no shutdown exit ip default-gateway 172.16.22.129 do wr |
| GWA SWITCH enable configure terminal vlan 260 name GWA-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR Guest/Waiting area (GWA) DEPARTMENT** switchport mode access switchport access vlan 260 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 260 switchport nonegotiate exit interface vlan 260 description **GWA DEPARTMENT VLAN INTERFACE** ip address 172.16.21.126 255.255.255.192 no shutdown exit ip default-gateway 172.16.21.65 do wr | BR-GWA SWITCH enable configure terminal vlan 320 name BR-GWA-DEPARTMENT exit vlan 999 name NATIVE-VLAN exit interface range FastEthernet 0/1-24 description **ACCESS PORTS FOR BR-GWA DEPARTMENT** switchport mode access switchport access vlan 320 no shutdown exit interface range GigabitEthernet 0/1-2 description **TRUNK LINKS** switchport mode trunk switchport trunk native vlan 999 switchport trunk allowed vlan 320 switchport nonegotiate exit interface vlan 320 description **BR-GWA DEPARTMENT VLAN INTERFACE** ip address 172.16.22.254 255.255.255.192 no shutdown exit ip default-gateway 172.16.22.193 do wr |
|  | Server Side SWITCH enable configure terminal vlan 999 name NATIVE-VLAN exit vlan 330 name SERVER-SIDE-NETWORK-DEPARTMENT exit interface range GigabitEthernet 1/0/1-13 description **ACCESS PORTS FOR OF Server Side Network (SERVER) DEPARTMENT ** switchport mode access switchport access vlan 320 no shutdown exit do wr |

## Subnetting and IP Addressing.

|  | DEPARTMENT | NETWORK ID | GATEWAYS / STARTING IP | LAST IP / VLAN IP | BROADCAST ID | SUBNET MASK |
| --- | --- | --- | --- | --- | --- | --- |
| HEAD OFFICE | MEDICAL LEAD OPERATION & CONSULTANCY SERVICES (MLOCS) DEPARTMENT | 172.16.200.0 | 172.16.200.1 | 172.16.200.62 | 172.16.200.63 | 255.255.255.192 |
|  | MEDICAL EMERGENCY AND REPORTING (MER) DEPARTMENT | 172.16.20.64 | 172.16.20.65 | 172.16.20.126 | 172.16.20.127 | 255.255.255.192 |
|  | MEDICAL RECORDS MANAGEMENT (MRM) DEPARTMENT | 172.16.20.128 | 172.16.20.129 | 172.16.20.190 | 172.16.20.191 | 255.255.255.192 |
|  | INFORMATION TECHNOLOGY (IT) DEPARTMENT | 172.16.20.192 | 172.16.20.193 | 172.16.20.254 | 172.16.20.255 | 255.255.255.192 |
|  | CUSTOMER SERVICE (CS) DEPARTMENT | 172.16.21.0 | 172.16.21.1 | 172.16.21.62 | 172.16.21.63 | 255.255.255.192 |
|  | HO GUEST/WAITING AREA (GWA) DEPARTMENT | 172.16.21.64 | 172.16.21.65 | 172.16.21.126 | 172.16.21.127 | 255.255.255.192 |
| BRANCH | NURSES & SURGERY OPERATIONS (NSO) DEPARTMENT | 172.16.21.128 | 172.16.21.129 | 172.16.21.190 | 172.16.21.191 | 255.255.255.192 |
|  | HOSPITAL LABS (HL) DEPARTMENT | 172.16.21.192 | 172.16.21.193 | 172.16.21.254 | 172.16.21.255 | 255.255.255.192 |
|  | HUMAN RESOURCE (HR) DEPARTMENT | 172.16.22.0 | 172.16.22.1 | 172.16.22.62 | 172.16.22.63 | 255.255.255.192 |
|  | MARKETING (MK) DEPARTMENT | 172.16.22.64 | 172.16.22.65 | 172.16.22.126 | 172.16.22.127 | 255.255.255.192 |
|  | FINANCE (FIN) DEPARTMENT | 172.16.22.128 | 172.16.22.129 | 172.16.22.190 | 172.16.22.191 | 255.255.255.192 |
|  | Guest/Waiting area (GWA) DEPARTMENT | 172.16.22.192 | 172.16.22.193 | 172.16.22.254 | 172.16.22.255 | 255.255.255.192 |

| R1-ROUTER | Gig 0/0/0 | 172.16.3.145 | 255.255.255.252 | M1-MULTILAYER-SWITCH Gig 1/0/1 |
| --- | --- | --- | --- | --- |
|  | Gig 0/0/1 | 172.16.3.154 | 255.255.255.252 | M2-MULTILAYER-SWITCH Gig 1/0/1 |
| R2-ROUTER | Gig 0/0/0 | 172.16.3.149 | 255.255.255.252 | M1-MULTILAYER-SWITCH Gig 1/0/2 |
|  | Gig 0/0/1 | 172.16.3.157 | 255.255.255.252 | M2-MULTILAYER-SWITCH Gig 1/0/2 |

| M1-MULTILAYER-SWITCH | Gig 1/0/1 | 172.16.3.146 | 255.255.255.252 | R1-ROUTER Gig 0/0/0 |
| --- | --- | --- | --- | --- |
|  | Gig 1/0/2 | 172.16.3.150 | 255.255.255.252 | R2-ROUTER Gig 0/0/0 |
| M2-MULTILAYER-SWITCH | Gig 1/0/1 | 172.16.3.153 | 255.255.255.252 | R1-ROUTER Gig 0/0/1 |
|  | Gig 1/0/2 | 172.16.3.158 | 255.255.255.252 | R1-ROUTER Gig 0/0/2 |

## Configuring Inter-VLAN Routing on the Multilayer switches (Switch Virtual Interface).

## Configuring Dedicated DHCP Server device to provide dynamic IP allocation.

## Configuring SSH for secure Remote access.

| MAIN HO PRIMARY ROUTER enable configure terminal ip domain-name sales.com crypto key generate rsa 1024 ip ssh version 2 username cisco secret cisco username admin secret admin line vty 0 15 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal do wr |
| --- | --- |
| MAIN HO SECONDARY ROUTER enable configure terminal do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal do wr |

| PRIMARY HO MULTILAYER SWITCH enable configure terminal do wr | PRIMARY BRANCH MULTILAYER SWITCH enable configure terminal do wr |
| --- | --- |
| SECONDARY HO MULTILAYER SWITCH enable configure terminal do wr | SECONDARY BRANCH MULTILAYER SWITCH enable configure terminal do wr |

| MLCOS SWITCH enable configure terminal do wr | NSO SWITCH enable configure terminal do wr |
| --- | --- |
| MER SWITCH enable configure terminal ip domain-name sales.com crypto key generate rsa 1024 ip ssh version 2 username cisco secret cisco username admin secret admin line vty 0 15 login local transport input ssh exit ip ssh time-out 60 ip ssh authentication-retries 3 do wr | HL SWITCH enable configure terminal do wr |
| MRM SWITCH enable configure terminal do wr | HR SWITCH enable configure terminal do wr |
| IT SWITCH enable configure terminal do wr | MK SWITCH enable configure terminal do wr |
| CS SWITCH enable configure terminal do wr | FIN ROOM SWITCH enable configure terminal do wr |
| GWA SWITCH enable configure terminal do wr | BR-GWA SWITCH enable configure terminal do wr |
|  | Server Side SWITCH enable configure terminal do wr |

## Configuring OSPF as the routing protocol.

| MAIN HO PRIMARY ROUTER enable configure terminal do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal do wr |
| --- | --- |
| MAIN HO SECONDARY ROUTER enable configure terminal do wr | SECONDARY CORE PRIMARY ROUTER enable configure terminal do wr |

| PRIMARY HO MULTILAYER SWITCH enable configure terminal do wr | PRIMARY BRANCH MULTILAYER SWITCH enable configure terminal do wr |
| --- | --- |
| SECONDARY HO MULTILAYER SWITCH enable configure terminal do wr | SECONDARY BRANCH MULTILAYER SWITCH enable configure terminal do wr |

| MLCOS SWITCH enable configure terminal do wr | NSO SWITCH enable configure terminal do wr |
| --- | --- |
| MER SWITCH enable configure terminal do wr | HL SWITCH enable configure terminal do wr |
| MRM SWITCH enable configure terminal do wr | HR SWITCH enable configure terminal do wr |
| IT SWITCH enable configure terminal do wr | MK SWITCH enable configure terminal do wr |
| CS SWITCH enable configure terminal do wr | FIN ROOM SWITCH enable configure terminal do wr |
| GWA SWITCH enable configure terminal do wr | BR-GWA SWITCH enable configure terminal do wr |
|  | Server Side SWITCH enable configure terminal do wr |

## Configuring NAT Overload (Port Address Translation PAT).

## Configuring Site-to-Site IPsec VPN.

## Configuring standard and extended Access Control Lists ACL.

## Configuring switchport security or Port-Security on the switches.

## Configuring WLAN or wireless network (Cisco Access Point).

## Host Device Configurations.

## Configuring ISP routers.

## Test and Verifying Network Communication.